CFD SIMULATION REPORT

Desuperheater — Counter-Current Flow Configuration

Transient DPM Evaporation Study

____________________________________________________________

Software: ANSYS Fluent 2026 R1 (Student Edition)

Date: 21 April 2026

Flow Configuration: Counter-Current (Nozzle spray in +Y, Steam inlet at top, Outlet at bottom)

# 1. Introduction

This report presents the CFD simulation of a desuperheater in counter-current flow configuration. In this arrangement, superheated dry steam enters from the top of the main pipe and flows downward (–Y direction), while liquid water is injected through a spray nozzle in the +Y direction (opposing the steam flow). The counter-current arrangement creates opposing momentum between the spray and steam, resulting in different mixing and evaporation characteristics compared to the co-current configuration.

## 1.1 Flow Configuration

Steam Inlet: Top of desuperheater (velocity inlet, –Y direction)

Steam Outlet: Bottom of desuperheater (pressure outlet)

Nozzle Spray: Counter-current to steam, injected in +Y direction from nozzle exit at Y = 233 mm

Water Inlet: Set as wall (blocks cold gas entry; water introduced via DPM only)

## 1.2 Key Difference from Co-Current

The only change from the co-current simulation is the reversal of steam inlet and outlet positions. The steam now enters from the top and exits from the bottom. The nozzle spray direction remains +Y (upward), creating a counter-current interaction where the spray opposes the steam flow. All other settings (geometry, mesh, materials, DPM injection, solver) remain identical.

# 2. Setup Summary (Identical to Co-Current Except Inlet/Outlet)

| Parameter | Value |
| --- | --- |
| Operating Pressure | 1,000,000 Pa (10 bar) |
| Steam Inlet | Top face — Velocity 30 m/s, T = 523 K, h2o = 1 |
| Steam Outlet | Bottom face — Pressure Outlet, Gauge P = 0 Pa |
| DPM Injection | Cone, 120 streams, 40 m/s, +Y axis, Cone Half Angle 15° |
| DPM Flow Rate | 0.1045 kg/s |
| Droplet Mean Diameter | 0.025 mm |
| Boiling Point | 453 K (at 10 bar) |
| Latent Heat | 2,015,000 J/kg |
| Turbulence | SST k-ω |
| Solver | Coupled, 2nd Order, Transient |
| Time Step | 0.0002 s × 2,500 steps = 0.5 s total |

# 3. Results — Counter-Current Flow

## 3.1 Residuals

Fig 3.1 — Scaled Residuals (Time = 0.5 s). Continuity ~1e-2, velocity 1e-4 to 1e-3, energy ~1e-8. More oscillatory than co-current due to opposing flow interaction.

## 3.2 Temperature Distribution

The temperature contour shows a highly localized cooling zone near the nozzle exit at the bottom of the pipe. The minimum temperature reaches approximately 364 K, significantly lower than in co-current flow (434 K). However, the cooling zone does not extend far up the pipe because the spray is opposed by the downward-flowing steam, limiting penetration.

| Fig 3.2a — Temperature contour (center plane, min 364 K) | Fig 3.2b — Temperature contour (cross-section) |
| --- | --- |

## 3.3 Outlet Temperature vs. Time

The outlet temperature (bottom of pipe) shows significant initial oscillations (470–525 K) during the first 0.1 s as the counter-current flow establishes itself. It then gradually stabilizes at approximately 508–510 K, representing a temperature reduction of ~13–15 K from the 523 K inlet. The convergence is slower and more oscillatory compared to co-current flow.

Fig 3.3 — Outlet Temperature vs. Flow Time (stabilized at ~508–510 K)

## 3.4 Evaporation Rate

The evaporation rate in counter-current flow shows a markedly different pattern from co-current flow. It peaks at approximately 0.007 kg/s during the initial transient, then decreases and oscillates around 0.0025 kg/s at steady state. This is approximately 3.5 times lower than the co-current evaporation rate (0.0088 kg/s), because the opposing steam flow limits the contact time between droplets and hot steam.

Fig 3.4 — Evaporation Rate vs. Flow Time (stabilized at ~0.0025 kg/s)

## 3.5 Pressure Distribution

The pressure contour shows a notable region of negative gauge pressure near the nozzle exit zone (down to −1,790 Pa), caused by the opposing momentum of the spray and steam. This creates a local low-pressure zone where the spray and steam collide.

| Fig 3.5a — Static pressure contour (center plane) | Fig 3.5b — Static pressure contour (cross-section) |
| --- | --- |

## 3.6 Pressure Drop vs. Time

The pressure drop shows extremely large initial oscillations (±80,000 Pa) that gradually dampen over 0.3 s and stabilize near 0 Pa. The initial instability is caused by the strong counter-current interaction as the spray and steam establish equilibrium. This is dramatically different from co-current flow where pressure drop oscillated only between 1,200–2,600 Pa.

Fig 3.6 — Pressure Drop vs. Flow Time (initial ±80 kPa oscillations, settles near 0)

## 3.7 Velocity Distribution

| Fig 3.7a — Velocity magnitude contour (center plane) | Fig 3.7b — Velocity magnitude contour (cross-section) |
| --- | --- |

## 3.8 Outlet Velocity vs. Time

The outlet velocity shows extremely large initial oscillations (0–100 m/s) that dampen over time and stabilize at approximately 30 m/s. The initial instability reflects the transient counter-current momentum exchange between the spray and steam.

Fig 3.8 — Outlet Velocity vs. Flow Time (initial 0–100 m/s oscillations, stabilizes at ~30 m/s)

## 3.9 Velocity Pathlines

Fig 3.9 — Pathlines colored by Velocity Magnitude (3D view)

## 3.10 Temperature Pathlines

Fig 3.10 — Pathlines colored by Static Temperature (3D view)

## 3.11 Turbulent Kinetic Energy

| Fig 3.11a — TKE contour (center plane) | Fig 3.11b — TKE contour (cross-section) |
| --- | --- |

## 3.12 Species (H₂O) Distribution

| Fig 3.12a — H₂O mass fraction (center plane) | Fig 3.12b — H₂O mass fraction (cross-section) |
| --- | --- |

## 3.13 Outlet H₂O vs. Time

Fig 3.13 — Outlet H₂O Mass Fraction (constant at 1.0)

## 3.14 DPM Particle Tracks

In counter-current flow, the particle tracks are concentrated near the nozzle exit zone (bottom third of the pipe). The spray cannot penetrate far against the opposing steam flow, so most droplets remain clustered in a compact region near the injection point. This is in stark contrast to co-current flow where particles spread throughout the entire pipe length.

Fig 3.14a — DPM particle tracks (wireframe view — spray concentrated at bottom)

Fig 3.14b — Particle tracks colored by Diameter (0–0.101 mm, clustered near nozzle)

Fig 3.14c — Particle tracks colored by Temperature

Fig 3.14d — Particle tracks colored by Velocity Magnitude

# 4. Results Summary — Counter-Current Flow

| Parameter | Value |
| --- | --- |
| Steam Inlet Temperature | 523 K (250°C) |
| Outlet Temperature (steady state) | ~508–510 K (235–237°C) |
| Temperature Reduction | ~13–15 K |
| Steam Inlet Velocity | 30 m/s (–Y direction, top to bottom) |
| Outlet Velocity (steady state) | ~30 m/s |
| Evaporation Rate (steady state) | ~0.0025 kg/s |
| Water Injection Rate | 0.1045 kg/s |
| Evaporation Efficiency | ~2.4% of injected water evaporated |
| Pressure Drop (steady state) | ~0 Pa (after initial transient) |
| Min Temperature in Spray Zone | ~364 K |
| Operating Pressure | 1,000,000 Pa (10 bar) |
| Simulation Time | 0.5 s (2,500 time steps × 0.0002 s) |

# 5. Comparison: Co-Current vs. Counter-Current

| Parameter | Co-Current | Counter-Current |
| --- | --- | --- |
| Steam Flow Direction | –Y to +Y (bottom to top) | +Y to –Y (top to bottom) |
| Spray Direction | +Y (same as steam) | +Y (opposing steam) |
| Outlet Temperature | ~508 K | ~508–510 K |
| Temperature Reduction | ~15 K | ~13–15 K |
| Evaporation Rate | ~0.0088 kg/s | ~0.0025 kg/s |
| Evaporation Efficiency | ~8.4% | ~2.4% |
| Spray Penetration | Full pipe length | Bottom ~1/3 of pipe only |
| Pressure Drop | 1,200–2,600 Pa (stable) | ±80,000 Pa initial, settles ~0 Pa |
| Outlet Velocity Stability | Stable (~29 m/s) | Large initial oscillations, settles ~30 m/s |
| Convergence Speed | Fast (∼0.03 s) | Slow (∼0.3 s) |
| Min Temp in Spray Zone | 434 K | 364 K |

# 6. Conclusions

1. The counter-current configuration produces a similar outlet temperature (~508–510 K) to the co-current case (~508 K), but achieves this through a fundamentally different mechanism. Counter-current flow creates intense local cooling near the nozzle (down to 364 K) but limited penetration, while co-current flow distributes cooling over the full pipe length.

2. The evaporation rate in counter-current flow (0.0025 kg/s) is approximately 3.5 times lower than in co-current flow (0.0088 kg/s). The opposing steam flow reduces droplet residence time in the hot zone and causes many droplets to be swept out of the pipe before fully evaporating.

3. Counter-current flow exhibits significantly more initial instability, with pressure oscillations of ±80,000 Pa and velocity oscillations of 0–100 m/s during the first 0.1–0.3 s. This reflects the strong momentum exchange when the spray and steam collide head-on.

4. The spray penetration in counter-current flow is limited to approximately the bottom third of the pipe, compared to the full pipe length in co-current flow. This concentrated spray zone creates more intense but spatially limited cooling.

5. For this desuperheater geometry and operating conditions, the co-current configuration is more effective for temperature reduction because it provides higher evaporation rates (3.5×), better spray distribution, and more stable flow conditions.

6. The counter-current configuration may be advantageous in applications requiring intense local cooling or where the spray needs to be contained within a specific zone, but it is less efficient for overall temperature reduction of the steam.